C Zero Sdn Bhd

OCPP 1.6J Integration Guide

Charger Hardware Vendor Onboarding — PlagSini EV Charging

Version 1.0
1 July 2026
Confidential — Partner Use Only

Document Version: 1.0

Date: 1 July 2026

Issued by: C Zero Sdn Bhd (PlagSini EV Charging Platform)

Audience: Charger hardware vendors integrating with PlagSini CSMS

1. Document control

| Field | Value |
| --- | --- |
| Operator | C Zero Sdn Bhd |
| Trading brand | PlagSini |
| Protocol | OCPP 1.6J (JSON over WebSocket) |
| Specification reference | Open Charge Alliance — OCPP 1.6 Edition 2 |
| Public spec URL | https://www.openchargealliance.org/protocols/ocpp-16/ |
| Production CSMS (WSS, recommended) | wss://charger.czeros.tech/ocpp/{charge_point_id} — port 443, TLS-terminated via nginx |
| Production CSMS (WS, fallback) | ws://charger.czeros.tech:9000/{charge_point_id} — port 9000, plain-text (only for firmware that cannot do TLS) |
| Testing CSMS | Same endpoints (test-mode charge points assigned per partner) |
| Primary contact | Aqid — aqidishak28@gmail.com |
| Technical contact | engineering@plagsini.com |

2. Protocol overview

PlagSini implements the Central System role per OCPP 1.6J.

Transport: WebSocket Secure (wss://) — TLS 1.2 or higher

Message format: JSON (per OCPP 1.6J specification)

Framing: OCPP Call / CallResult / CallError structure

Sub-protocol header required: Sec-WebSocket-Protocol: ocpp1.6

Charge points connect as client, PlagSini CSMS acts as server

3. Connection setup

3.1 Endpoint

PlagSini exposes two OCPP endpoints. Choose based on charger firmware capability:

Option A — WSS (encrypted, recommended for production)

wss://charger.czeros.tech/ocpp/{charge_point_id}

Port: 443 (standard HTTPS)

TLS terminated at Nginx reverse proxy

Firmware must support TLS 1.2 or higher

Server certificate: Let's Encrypt (widely trusted CA)

Option B — WS (plain, fallback for firmware without TLS)

ws://charger.czeros.tech:9000/{charge_point_id}

Port: 9000 (direct to CSMS container)

Plain-text WebSocket, no TLS

Suitable for firmware that does not implement TLS

Not recommended for production over untrusted networks — use only if firmware constraint requires it

Where {charge_point_id} is the unique identifier assigned by PlagSini during provisioning (Section 4). Examples:

wss://charger.czeros.tech/ocpp/TRANSSEMI-22KW-001
ws://charger.czeros.tech:9000/TRANSSEMI-22KW-001

3.2 TLS / certificate

Server certificate: Let's Encrypt (widely trusted CA — no additional root cert needed on charger firmware if it uses standard CA bundle)

Minimum TLS version: 1.2

Cipher suites: ECDHE-based, forward secrecy

If your firmware runs on an older embedded platform without recent CA roots, request the ISRG Root X1 root certificate from us and pre-load it.

3.3 Authentication

PlagSini enforces per-charger token authentication in production. Two authentication modes are supported (choose based on firmware capability):

Mode A — HTTP Basic Auth in WebSocket handshake (recommended)

Authorization: Basic <base64(charge_point_id:token)>

Mode B — Bearer token subprotocol negotiation

Sec-WebSocket-Protocol: ocpp1.6
Authorization: Bearer <token>

The token is a random 64-character string issued to your team per charger, kept secret. Rotate on request; no need to hardcode in mass-production firmware if a provisioning mechanism is available.

3.4 Connection flow

Charger                              PlagSini CSMS
       |                                       |
       |----- WebSocket upgrade ---------------|
       |         + Authorization header        |
       |         + Sec-WebSocket-Protocol      |
       |                                       |
       |<---- 101 Switching Protocols ---------|
       |                                       |
       |----- BootNotification ----------------|
       |                                       |
       |<---- BootNotification.conf -----------|
       |         (interval, status: Accepted)  |
       |                                       |
       |----- Heartbeat (every N seconds) ---->|
       |<---- Heartbeat.conf ------------------|
       |                                       |

If BootNotification returns status: Rejected, charger should retry after the returned interval. If status: Pending, use TriggerMessage to re-send.

4. Charger provisioning process

Before your charger can connect, PlagSini must provision the unit in our database.

4.1 What you send us

Charger model & manufacturer

Firmware version

Rated power (kW)

Connector type (Type 2, CCS2, CHAdeMO, GB/T)

Number of connectors

Charger serial number (for record only)

Preferred charge_point_id (optional — otherwise we generate one)

4.2 What we send back

Assigned charge_point_id

Authentication token

Endpoint URL confirmation

Test / production flag

4.3 Firmware configuration on your side

Configure the charger via its local UI, CLI, or configuration file with:

| Config key | Value |
| --- | --- |
| OcppServerUrl (or vendor-equivalent) | wss://charger.czeros.tech/ocpp (WSS) *or* ws://charger.czeros.tech:9000 (WS) |
| ChargePointId | (from provisioning) |
| AuthorizationKey (basic-auth token) | (from provisioning) |
| HeartbeatInterval | 60 seconds (recommended) |
| MeterValueSampleInterval | 60 seconds (recommended) |
| WebSocketPingInterval | 30 seconds |
| ConnectionTimeOut | 60 seconds |

5. Supported OCPP messages

PlagSini implements the full OCPP 1.6J Core Profile plus commonly requested Firmware and Remote profiles.

5.1 Charger → CSMS (initiated by charger)

| Message | Support | Notes |
| --- | --- | --- |
| BootNotification | ✅ Required on connect | Response contains interval, status |
| Heartbeat | ✅ Required periodic | Every 60 seconds recommended |
| StatusNotification | ✅ Required on state change | Connector state machine |
| StartTransaction | ✅ | Session start, returns transactionId |
| StopTransaction | ✅ | Session end, meter reading |
| MeterValues | ✅ | Periodic energy / power reporting |
| Authorize | ✅ | RFID / token validation |
| DataTransfer | ✅ | Vendor-specific extensions |
| FirmwareStatusNotification | ✅ | Firmware update progress |
| DiagnosticsStatusNotification | ✅ | Diagnostics upload progress |

5.2 CSMS → Charger (initiated by PlagSini)

| Message | Support | Notes |
| --- | --- | --- |
| RemoteStartTransaction | ✅ | Trigger session start from app / dashboard |
| RemoteStopTransaction | ✅ | Trigger session stop |
| ChangeConfiguration | ✅ | Runtime config update |
| GetConfiguration | ✅ | Read charger configuration keys |
| ChangeAvailability | ✅ | Set connector Operative / Inoperative |
| Reset | ✅ | Soft / Hard reboot |
| UnlockConnector | ✅ | Release cable lock |
| TriggerMessage | ✅ | Force charger to send a specific message |
| ClearCache | ✅ | Clear local authorisation cache |
| UpdateFirmware | ✅ | OTA firmware download instruction |
| GetDiagnostics | ✅ | Upload diagnostics file |
| ReserveNow | ✅ | Reservation support |
| CancelReservation | ✅ | Cancel active reservation |

If your firmware does not support one of the CSMS-initiated messages, PlagSini gracefully handles the error response — the charger simply returns NotImplemented or NotSupported per spec.

6. Sample message sequences

All examples are OCPP 1.6J JSON on the WebSocket.

6.1 Boot on connect

// Charger → CSMS
[2, "unique-message-id-1", "BootNotification", {
  "chargePointVendor": "Transsemi",
  "chargePointModel": "22kW-AC",
  "chargePointSerialNumber": "TS22-2026-0001",
  "firmwareVersion": "1.0.0"
}]

// CSMS → Charger
[3, "unique-message-id-1", {
  "status": "Accepted",
  "currentTime": "2026-07-01T08:30:00Z",
  "interval": 60
}]

6.2 Heartbeat

// Charger → CSMS
[2, "msg-id-2", "Heartbeat", {}]

// CSMS → Charger
[3, "msg-id-2", { "currentTime": "2026-07-01T08:31:00Z" }]

6.3 Status notification (plug in)

// Charger → CSMS
[2, "msg-id-3", "StatusNotification", {
  "connectorId": 1,
  "errorCode": "NoError",
  "status": "Preparing",
  "timestamp": "2026-07-01T08:35:00Z"
}]

// CSMS → Charger
[3, "msg-id-3", {}]

6.4 Full charging session

// 1. User authorises (or dashboard triggers RemoteStartTransaction)
[2, "id-4", "Authorize", { "idTag": "APP_USER_001" }]
[3, "id-4", { "idTagInfo": { "status": "Accepted" } }]

// 2. StartTransaction
[2, "id-5", "StartTransaction", {
  "connectorId": 1,
  "idTag": "APP_USER_001",
  "meterStart": 12345,
  "timestamp": "2026-07-01T08:36:00Z"
}]
[3, "id-5", {
  "transactionId": 101,
  "idTagInfo": { "status": "Accepted" }
}]

// 3. MeterValues (every 60s during session)
[2, "id-6", "MeterValues", {
  "connectorId": 1,
  "transactionId": 101,
  "meterValue": [{
    "timestamp": "2026-07-01T08:37:00Z",
    "sampledValue": [
      { "value": "12500", "measurand": "Energy.Active.Import.Register", "unit": "Wh" },
      { "value": "22000", "measurand": "Power.Active.Import", "unit": "W" }
    ]
  }]
}]
[3, "id-6", {}]

// 4. StopTransaction
[2, "id-7", "StopTransaction", {
  "transactionId": 101,
  "idTag": "APP_USER_001",
  "meterStop": 22345,
  "timestamp": "2026-07-01T09:36:00Z",
  "reason": "EVDisconnected"
}]
[3, "id-7", { "idTagInfo": { "status": "Accepted" } }]

6.5 Server-initiated Remote Start

// CSMS → Charger
[2, "srv-id-1", "RemoteStartTransaction", {
  "connectorId": 1,
  "idTag": "APP_USER_001"
}]

// Charger → CSMS
[3, "srv-id-1", { "status": "Accepted" }]

// Charger then follows the normal session flow (Authorize → StartTransaction → ...)

6.6 Server-initiated Reset

// CSMS → Charger
[2, "srv-id-2", "Reset", { "type": "Soft" }]

// Charger → CSMS
[3, "srv-id-2", { "status": "Accepted" }]

7. Charger state machine

PlagSini expects the charger to report state via StatusNotification with one of these values (per OCPP spec):

| Status | Meaning |
| --- | --- |
| Available | Idle, ready for use |
| Preparing | Plug inserted, waiting for authorisation |
| Charging | Session active, energy flowing |
| SuspendedEV | Session paused by vehicle (SOC full etc.) |
| SuspendedEVSE | Session paused by charger |
| Finishing | Session ending, plug still inserted |
| Reserved | Reserved for a user |
| Unavailable | Manually disabled or configured Inoperative |
| Faulted | Hardware error |

Report transitions promptly so the dashboard reflects live state.

8. Security requirements

8.1 TLS

Minimum TLS 1.2 (TLS 1.3 preferred)

Server certificate verification MUST be enabled on charger firmware

Do not disable certificate validation in production

8.2 Authentication

Never share tokens across chargers — one token per unit

Store token in secure firmware storage (encrypted, not plain-text file)

Rotate token on physical decommission of charger

8.3 Message integrity

Charger must handle CallError responses gracefully — do not treat as fatal

Reconnect on WebSocket close with exponential backoff (start 2s, cap 60s)

Do not spam server on error — respect the returned interval

9. Testing checklist (docking certification)

Complete the following checks during joint docking test to certify a charger model:

[ ] WebSocket handshake succeeds with correct token

[ ] BootNotification sent within 30 seconds of connection

[ ] BootNotification.conf received and interval respected

[ ] Heartbeat sent at configured interval

[ ] StatusNotification sent on plug-in (Available → Preparing)

[ ] Authorize request handled by app-side auth (RFID or app trigger)

[ ] StartTransaction returns valid transactionId

[ ] MeterValues sent at configured interval during session

[ ] Values include Energy.Active.Import.Register (Wh) and Power.Active.Import (W)

[ ] StopTransaction sent on plug-out with correct meterStop

[ ] RemoteStartTransaction (server → charger) accepted and executes

[ ] RemoteStopTransaction (server → charger) accepted and executes

[ ] Reset (Soft) command reboots charger cleanly

[ ] UnlockConnector command releases cable

[ ] ChangeAvailability (Inoperative → Operative) works

[ ] Reconnect on network loss with backoff (verified by pulling ethernet for 30s)

[ ] Vendor DataTransfer messages (if any) documented and accepted

10. Troubleshooting common issues

| Symptom | Likely cause | Resolution |
| --- | --- | --- |
| WebSocket 401 Unauthorized | Wrong token or wrong Authorization header format | Verify token, check header case-sensitivity |
| WebSocket 400 Bad Request | Missing Sec-WebSocket-Protocol: ocpp1.6 header | Add the sub-protocol header |
| TLS handshake fails | Missing / expired CA bundle on charger | Update charger firmware CA store to include Let's Encrypt ISRG Root X1 |
| BootNotification rejected | Charger not provisioned in PlagSini DB | Contact PlagSini for provisioning |
| No response to CSMS commands | Charger not sending Heartbeat / stale WebSocket | Check network, firewall, keepalive settings |
| MeterValues missing key measurands | Incomplete OCPP profile support | Enable Energy.Active.Import.Register and Power.Active.Import at minimum |
| Frequent reconnects | Firewall dropping idle WebSockets | Reduce WebSocketPingInterval to 20 seconds |
| Charger shows Offline in dashboard | Missed heartbeat for > 5 minutes | Verify network stability |

11. Service level & availability

| Metric | Target |
| --- | --- |
| CSMS availability | 99.5% (excluding scheduled maintenance) |
| WebSocket accept latency | ≤ 500 ms (95th percentile) |
| BootNotification response latency | ≤ 500 ms |
| CDR generation after StopTransaction | ≤ 60 s |
| Scheduled maintenance | Announced 48 hours in advance, off-peak hours (MYT) |

12. Contact & support

For technical questions, provisioning requests, or docking coordination:

Project Lead: Aqid — aqidishak28@gmail.com

Engineering: engineering@plagsini.com

Production CSMS (WSS): wss://charger.czeros.tech/ocpp/{charge_point_id}

Production CSMS (WS): ws://charger.czeros.tech:9000/{charge_point_id}

Status endpoint: https://charger.czeros.tech/health

Company: C Zero Sdn Bhd

Typical response time: within 1 business day (MYT).

13. Change log

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 1 July 2026 | C Zero Engineering | Initial release |

This document is issued by C Zero Sdn Bhd and remains the intellectual property of the company. Confidential — for partner use under signed NDA where applicable.